ICST Ambisonics

Workshop

Institute for Computer Music and Sound Technology (ICST)

Zurich University of the Arts (ZHdK)

Tuesday 19 May 2026  ·  13:00-17:00  ·  Language: English

# 1  Learning Objectives

After this workshop, participants will be able to:

•  explain the Ambisonics signal chain from recording to delivery

•  distinguish A-Format (PCM audio) from B-Format (mathematical/HOA)

•  carry out a Zylia ZM-1 HOA recording and A-to-B conversion workflow

•  position sound sources in 3D space using the ICST AmbiEncoder_64

•  describe and apply the three stages: Encoding, Processing, Decoding

•  decode Ambisonics binaurally and for speaker systems

•  know where MaxMSP, Csound and Max for Live fit into an Ambisonics workflow

Participant preparation: Reaper is pre-installed on all studio computers (v7+, incl. ICST plugins). Please bring your own headphones. Download the pre-configured Reaper session from ambisonics.ch before the workshop.

# 2  Workshop Schedule (4 hours)

| Time | Content | Dur. | Block |
| --- | --- | --- | --- |
| 13:00-13:10 | Getting started — demo & introduction | 10' | - |
| 13:10-13:40 | Overall Workflow: How to work in Ambisonics | 30' | A |
| 13:40-14:20 | How to record Ambisonics (Zylia ZM-1) | 40' | B |
| 14:20-14:35 | Composers Workflow: 3 stages | 15' | B |
| 14:35-14:45 | Break | 10' | - |
| 14:45-15:15 | Reaper Workflow: Encoding & Processing | 30' | C |
| 15:15-16:15 | Practical session (guided exercise) | 60' | C |
| 16:15-17:00 | Composer Discussion & Outlook | 45' | G |

| ID | Block | Content |
| --- | --- | --- |
| A | Overall Workflow | How to work in Ambisonics. The big picture: signal chain from recording to delivery, formats, why Ambisonics. |
| B | How to Record | Zylia ZM-1 HOA recording in Reaper. A-Format vs B-Format. Recording workflow: capture, A-to-B, listen, FX, master. Composers Workflow: the 3 stages (Encoding / Processing / Decoding). |
| C | Reaper Workflow | Hands-on: open Reaper session, encoding (ICST AmbiEncoder_64), processing (FuMa to AmbiX, reverb, distance, automation), decoding (MultiDecoder, Binaural). |
| D | External Tools | Workflows outside Reaper: MaxMSP, Csound (Cabbage.app), Max for Live. |
| E | Csound / Cabbage | Csound opcodes for spatial audio. Additive synth & random spatialisation example in Cabbage. |
| F | MaxMSP | ICST Ambisonics externals for MaxMSP. ACT-Tool example: real-time spatialisation and automation. |
| G | Composer Discussion | Guided discussion on compositional strategies, participant presentations, Q&A, and outlook. |

# A  Overall Workflow  —  How to Work in Ambisonics

Ambisonics is a fully spherical, channel-format-independent audio system. Unlike stereo (left/right only), Ambisonics encodes the entire sound field — 360 degrees horizontally and vertically — into a mathematical representation called B-Format. This B-Format can then be decoded for any playback system: headphones, a 4-speaker square, a 24-speaker dome, or a YouTube 360 video.

The key insight: you produce once and play back anywhere. The same B-Format master works for headphones, loudspeakers and streaming without needing to re-mix.

## The Ambisonics Signal Chain

1. Capture / Source  Microphone array (Zylia ZM-1) or virtual source (synth, plugin)

2. A-to-B Conversion  Raw capsule data (A-Format PCM) → B-Format (spherical harmonics) via Zylia App

3. Encoding  Position a sound source in 3D space: azimuth, elevation, distance via ICST AmbiEncoder

4. Processing  EQ, reverb, rotation, movement automation on the HOA bus

5. Decoding  B-Format → playback format: binaural (HRTF), speakers (.spk config)

6. Delivery  Render the B-Format master (never the decoder output!)

Golden rule: always render the B-Format master track — never the decoder output. The decoder is for monitoring only. One B-Format master works everywhere.

## Why Not Just Use Stereo?

•  Stereo encodes only the left/right axis. Height and depth are lost.

•  Ambisonics encodes the full sphere: azimuth (0-360 deg), elevation (-90 to +90 deg), distance.

•  Binaural decoding from Ambisonics gives front/back and height cues that stereo cannot provide.

•  A single B-Format file is format-agnostic: decode it for headphones, a concert hall, or an installation — without remixing.

## HOA Orders — Spatial Resolution

1st Order (FOA) — 4 channels: Good starting point. Coarse directional resolution. Microphones: Zoom H3-VR, Sennheiser Ambeo.

2nd Order — 9 channels: Noticeably sharper localisation, especially for frontal sources.

3rd Order (HOA) — 16 channels: High spatial resolution. Reference for production. Microphone: Zylia ZM-1.

## Formats: FuMa and AmbiX

•  FuMa (Furse-Malham): older standard, channel order W X Y Z, MaxN normalisation. Common in legacy recordings (SoundField mic).

•  AmbiX (ACN/SN3D): current standard, channel order W Y Z X (ACN 0-3), SN3D normalisation. Used by Zylia, ICST plugins, IEM, Ambix.

•  Best practice: always work and archive in AmbiX. Convert FuMa material before processing (see Block C).

## Delivery Formats

| Target | Format | Notes |
| --- | --- | --- |
| Archive / Master | Multichannel WAV, AmbiX, 48 kHz / 32-bit float | Unmodified B-format master -- the source of truth |
| Binaural | 2-channel WAV | Streaming, preview, headphones -- decoded with HRTF |
| Speaker ring | N-channel WAV | Performance, installation -- decoded for speaker layout |
| YouTube 360 | Binaural + spatial metadata | Spatial Media Metadata Tool required |

# B  How to Record Ambisonics  —  Zylia ZM-1

## Microphone: Zylia ZM-1

•  19-capsule spherical array (omnidirectional capsules)

•  Outputs 19 raw channels (A-Format PCM) via USB-C

•  Zylia App converts A-Format → 3rd-order HOA (16 channels, AmbiX)

•  No external power supply, no preamp needed — plug-and-play

The Zylia ZM-1 produces A-Format (19-channel PCM audio). This is NOT yet Ambisonics — it must be converted to B-Format first using the Zylia App.

## Key Concept: A-Format vs B-Format

This is the most important conceptual distinction in HOA recording:

|  | A-Format | B-Format |
| --- | --- | --- |
| What is it? | Raw capsule recordings — standard PCM audio | Spherical harmonic representation of the sound field |
| Type | Normal audio (like any microphone) | Mathematical / encoded format (not normal audio) |
| Channels | 19 channels (Zylia ZM-1) | 4 ch (1st order) / 16 ch (3rd order, HOA) |
| Can you listen? | Yes — but it sounds strange (no room, just capsules) | Yes via decoder (AmbiX W = omni), but needs decoding for spatial playback |
| When to use | Archive raw data — keep for future re-conversion | Working format: all processing, mixing and delivery |
| Conversion | Zylia App: A-Format (PCM) → B-Format (AmbiX) | JSFX or Lua: FuMa → AmbiX, if using legacy material |

Think of A-Format as the raw ingredient and B-Format as the cooked dish. You always archive the raw ingredient (A-Format) and work with the cooked dish (B-Format). If a better recipe (better conversion algorithm) becomes available in the future, you can re-cook from the raw ingredient.

## Recording Workflow (Praxis with Zylia → Main Reaper)

| # | Step | Action | Result |
| --- | --- | --- | --- |
| 1 | Short Recording Session | Set up Zylia ZM-1, check levels (-18 dBFS), record 30-60 seconds | 19-channel A-Format PCM file |
| 2 | A-to-B Converting | Open Zylia App, import A-Format, export as 3rd-order HOA | 16-channel B-Format AmbiX WAV |
| 3 | Listening B-Format | Import into Reaper, route to HOA Bus + AmbiDecoder, listen binaural | Spatial orientation check: front = front? |
| 4 | Upsampling & FX | EQ, reverb on HOA Bus, distance automation on encoder | Processed B-Format on HOA Bus |
| 5 | Mastering B-Format | Render HOA Render Bus: 16 ch, 48 kHz, 32-bit float WAV | B-Format Master -- ready for delivery |

### 1. Short Recording Session

•  Set up Zylia ZM-1 at approx. 1.5 m height (ear level)

•  Note orientation: mark which direction is 'front' on the mic (sticker, compass, photo)

•  Target level: -18 dBFS on all 19 channels — check the Zylia App's per-channel meters

•  Headroom: minimum 12 dB below 0 dBFS. Transients in HOA microphones can be unpredictable.

•  Record at least 30 seconds of ambience before and after the material you need

### 2. A-to-B Converting

•  Open Zylia App, import the A-Format WAV file (19 channels)

•  Select output: 3rd-order HOA, AmbiX format, 48 kHz / 32-bit float

•  Export → you receive a 16-channel B-Format WAV file

•  Keep the original A-Format file archived — re-conversion is always possible

### 3. Listening B-Format

•  Import the 16-channel WAV into the pre-configured Reaper session (HOA Bus track)

•  Put on headphones — the Binaural Monitor track is active

•  Listen: does 'front' sound like front? Does the recording have height?

•  A mono click from directly in front of the mic should appear exactly in front binaurally

### 4. Upsampling & FX

•  EQ on the HOA Bus: roll off below 60 Hz (room rumble) and above 18 kHz (conversion artefacts)

•  Reverb on the HOA Bus (ReaVerb): sits in the Ambisonics space, decoded with B-Format

•  Distance automation: use ICST AmbiEncoder Param 13 to add depth to encoded sources

•  Volume/gain: treat the HOA Bus like a stereo mix bus — keep peaks below -3 dBFS

### 5. Mastering B-Format

•  Render via File → Render → select HOA Render Bus

•  Settings: WAV, 48 kHz, 32-bit float, 16 channels, no dithering

•  This is the B-Format master — the archive and source for all delivery formats

•  Verify: open the rendered file in a new session, listen binaural, confirm it matches

## Example Recording: Gewitter (Thunderstorm)

A thunderstorm recording demonstrates why HOA is superior to stereo for ambient sound:

•  Low-frequency thunder: HOA captures the full envelopment — it comes from everywhere, not just left/right

•  Rain on the ground: elevation is captured — drops fall from above

•  Wind direction: the Zylia ZM-1 captures a rotating sound field — you can hear the storm moving

•  Lightning: a sudden directional transient — precise localisation in 3D space

In stereo: only a left/right smear. In Ambisonics: full spherical immersion.

## Microphone Placement

•  Height: 1.5 m = ear level (natural perspective). Lower = 'crawling' perspective. Higher = overlooking.

•  Orientation: always document which direction is 'front'. Use a compass or photo. Wrong orientation = reversed decoding.

•  Distance to source: closer = more direct sound, drier. Further = more room, more diffusion.

•  Reflections: hard walls are very present in HOA — position carefully, listen before committing

•  Wind: always use blimp outdoors. Enable LF roll-off below 80 Hz.

## Fieldrecording Checklist

Before recording:

☐  Acoustic survey: reflections, noise sources, wind direction

☐  Mic position documented (height, orientation, distance to main source)

☐  Level check: all 19 channels at approx. -18 dBFS

☐  Binaural monitoring active and verified (front = front)

☐  Metadata template ready (location, date, take number, weather)

After recording:

☐  A-to-B conversion in Zylia App

☐  B-Format file imported into Reaper and verified binaurally

☐  A-Format (raw 19 channels) archived to backup drive

## Composers Workflow — The Three Stages

Whether you are working with a field recording, a synthesizer, or a live instrument — every Ambisonics production goes through the same three stages:

| Stage | Question | In practice |
| --- | --- | --- |
| 1 Encoding | Where is the sound in 3D space? | Azimuth, elevation, distance — via ICST AmbiEncoder_64 or Lua script |
| 2 Processing | How does the sound move and interact with the room? | Rotation, movement automation, reverb on HOA bus, distance simulation, FuMa/AmbiX conversion |
| 3 Decoding | How do I play this on my audio system? | ICST AmbiDecoder: Binaural (HRTF), speaker ring, MultiDecoder mode |

These three stages are independent. You can encode a sound, process it over days, and decode it for a completely different system at the end. The B-Format is the neutral, format-agnostic medium between them.

## Typical Errors and Solutions

| Problem | Cause | Solution |
| --- | --- | --- |
| Sound reversed (front/back) | Mic orientation not documented | Test and note orientation before recording |
| Comb filtering / no direction | Faulty A-to-B conversion | Check spectral artefacts, redo conversion in Zylia App |
| Clipping on channels | Single capsule overload | Monitor all 19 channels separately, reduce gain |
| Wind noise in low freq. | No wind protection | LF roll-off below 80 Hz, use blimp or fur |
| Elevation not perceived | FOA (4 ch) loaded instead of HOA (16 ch) | Check channel count in Reaper track properties |
| Wrong B-format in Reaper | FuMa ordering instead of AmbiX | Apply FuMa->AmbiX JSFX converter (see Block C) |

# C  Reaper Workflow  —  Encoding · Processing · Decoding

## Open Reaper Session

•  Open zylia_recording_example.RPP (available at ambisonics.ch)

•  Session is pre-configured: Zylia track, HOA Bus, AmbiDecoder, Binaural Monitor

•  Screensets: Ctrl+Alt+1 (Recording)  ·  Ctrl+Alt+2 (Mixing)  ·  Ctrl+Alt+3 (Decoding)

## Workflow Overview in Reaper

•  Source tracks (mono/stereo) → ICST AmbiEncoder_64 → HOA Bus (16 ch)

•  Field recording (16 ch B-Format) → directly onto HOA Bus

•  HOA Bus → FX chain (EQ, reverb) → ICST AmbiDecoder → Binaural Monitor

•  HOA Render Bus: parallel bus, muted during session, active only for offline render

## Encoding — Where is the Sound in 3D Space?

The ICST AmbiEncoder_64 (VST3) positions any mono or stereo source in the Ambisonics sound field. Key parameters for Lua automation:

| Parameter | Index | Range | Meaning |
| --- | --- | --- | --- |
| Azimuth | 11 | 0-1 -> -180 to +180 deg | Horizontal angle: 0 = front, 0.25 = right, 0.5 = rear, 0.75 = left |
| Elevation | 12 | 0-1 -> -90 to +90 deg | Vertical angle: 0.5 = horizon, 1.0 = directly above |
| Distance | 13 | 0 = near / 1 = far | Depth: attenuates level and high frequencies with distance |

•  Encoding example: voice at azimuth -45 deg, elevation +20 deg → Lua script: icst_ambi_encoding_voice.lua

•  Physics-based trajectory: train flyby (icst_ambi_zug_langsam.lua, icst_ambi_zug_schnell.lua)

•  3D automation: helicopter flyover (icst_ambi_kopter.lua) — azimuth, elevation, distance change simultaneously

## Processing — Rotation, Movement, Reverb, Spatial Sound

### FuMa to AmbiX Conversion

Legacy recordings (SoundField microphone, older productions) are often in FuMa format. The ICST JSFX plugin converts them to AmbiX automatically:

| FuMa input | AmbiX output (ACN) | Channel function | Gain correction |
| --- | --- | --- | --- |
| Ch1 = W | ACN 0 = W | Omnidirectional | x sqrt(2) = +3.01 dB |
| Ch2 = X | ACN 3 = X | Front / Rear | x 1/sqrt(3) = -4.77 dB |
| Ch3 = Y | ACN 1 = Y | Left / Right | x 1/sqrt(3) = -4.77 dB |
| Ch4 = Z | ACN 2 = Z | Top / Bottom | x 1/sqrt(3) = -4.77 dB |

•  Lua installer: icst_fuma_to_ambix_foa.lua — writes JSFX to Reaper and loads it on the selected track

•  In Reaper: Actions → Load new Script → icst_fuma_to_ambix_foa.lua

### Reverb on the HOA Bus

•  Reverb must be on the HOA Bus, not on individual source tracks

•  Reason: reverb on the bus stays in the Ambisonics space — it will be decoded correctly

•  Reverb on a source track before the encoder = the reverb is pinned to the encoded position, not the room

•  Start dry (~10% wet) — HOA reverb is very enveloping and can quickly overwhelm the mix

### Distance Simulation

•  Encoder Param 13: attenuates level and high frequencies as distance increases

•  Near (0): present, direct, full HF — sounds like a close-mic'd source

•  Far (1): quieter, duller, more diffuse — sounds like a distant source in a room

•  Combine with reverb wet level for depth: far sources get more reverb, near sources less

## Decoding — How Do I Play This on My Audio System?

### Binaural Decoding (Headphones)

•  ICST AmbiDecoder with HRTF preset → 2-channel output → Binaural Monitor track

•  HRTF = Head-Related Transfer Function: simulates how a sound reaches the ears from any direction

•  Result: spatial audio on any pair of headphones, no special hardware needed

### MultiDecoder Mode

•  The ICST AmbiDecoder supports multiple simultaneous decoder outputs

•  Example: binaural monitoring + speaker ring output active at the same time (with separate buses)

•  Use case: switching between headphone check and speaker check without changing the session

•  Never mix decoder outputs: each output channel set needs its own physical output

# D  External to Reaper  —  MaxMSP · Csound · Max for Live

Reaper is not the only environment for Ambisonics. The following tools are commonly used by composers and researchers working with spatial audio — each with a different approach and different strengths.

## Workflow with MaxMSP

•  ICST Ambisonics externals: ambicontrol, ambiencode, ambidecode — freely available from ICST

•  Real-time spatialisation of any audio source: microphone input, synth, sample player

•  OSC control: the ICST AmbiEncoder in Reaper also accepts OSC — MaxMSP can drive it remotely

•  Strength: generative and algorithmic spatialisation, live electronics, interactive installations

•  Typical use: spatial sound design in real time, custom GUIs for performer control

## Workflow with Csound (Cabbage.app)

•  Csound opcodes: bformenc1 (encode to B-Format), bformdec2 (decode from B-Format)

•  Cabbage: a VST wrapper for Csound — run Csound instruments as plugins in any DAW

•  Strength: precise mathematical control of the sound field, additive synthesis, granular synthesis

•  Typical use: generative spatial composition, research, physically-based sound models

## Workflow with Max for Live

•  Max for Live = MaxMSP integrated inside Ableton Live

•  Ambisonics in Max for Live: use ICST or IRCAM externals inside Ableton

•  Strength: combines Ableton's production workflow (clips, session view) with spatial audio

•  Typical use: live performance with Ableton + Ambisonics spatialisation, studio production

Content blocks D, E and F go beyond the 4-hour framework. They are introduced here as an overview and will be covered in a follow-up workshop.

# E  Csound / Cabbage

Csound is a programmable audio synthesis language. Cabbage wraps Csound instruments as VST3 plugins, making them usable in Reaper and any other DAW. For Ambisonics, Csound gives the composer direct control over the spherical harmonic encoding and decoding — down to the mathematical level.

## Key Csound Opcodes for Ambisonics

•  bformenc1: encodes a mono audio signal into B-Format (1st-3rd order). Inputs: audio signal, azimuth, elevation. Output: HOA channels.

•  bformdec2: decodes B-Format to a speaker array. Inputs: HOA channels, decoder mode. Output: speaker feeds.

•  ambiamb / ambienc: alternative encoding opcodes for specific use cases

## Example: Additive Synth & Random Spatialisation (Cabbage)

This Cabbage example demonstrates:

•  An additive synthesizer generates partials (sine waves) in real time

•  Each partial is independently spatialised using bformenc1 with randomised azimuth and elevation

•  The azimuth and elevation of each partial drift randomly over time — creating a moving cloud of sound

•  The combined B-Format signal is decoded binaurally for headphone monitoring

This technique is used in spatial electroacoustic composition: instead of placing a sound object at a fixed point, the composer places a texture that moves and evolves in 3D space.

# F  MaxMSP — ICST Ambisonics Externals

MaxMSP with the ICST Ambisonics externals provides a complete real-time Ambisonics environment. Unlike a DAW, Max allows algorithmic and generative control of spatialisation — positions can be driven by data, gestures, sensors or other audio signals.

## ICST Ambisonics Externals

•  ambiencode~: encodes a mono audio signal to B-Format. Message input for azimuth, elevation, distance.

•  ambidecode~: decodes B-Format to speaker or binaural output.

•  ambicontrol: GUI panel for manual spatialisation control

•  Available at: icst-ambisonics.github.io

## Example: ACT-Tool

The ACT-Tool (Ambisonics Composition Tool) is a MaxMSP patch developed at ICST:

•  Provides a 3D visual interface for placing and animating sound sources

•  Sources can be moved manually (mouse/trackpad) or driven algorithmically (LFOs, envelopes, data streams)

•  OSC output: sends azimuth, elevation and distance to any Ambisonics encoder — including the ICST AmbiEncoder in Reaper

•  Use case: live spatial performance, interactive installation, composer-tool for trajectory design

The ACT-Tool can be used as a 'spatial score': define the movement of sound sources visually, then record the OSC output as automation in Reaper.

# Practical Session  (60')  —  15:15-16:15

Guided exercise with the pre-configured Reaper session. Work through all six steps in order.

### Step 1 — Open session and verify routing  (5')

•  Open zylia_recording_example.RPP

•  Zylia ZM-1 → HOA Bus (16 ch). HOA Bus → AmbiDecoder → Binaural Monitor (2 ch).

•  Put on headphones. Confirm spatial audio is working.

### Step 2 — Listen and explore the B-Format recording  (5')

•  Play the pre-loaded Zylia B-Format recording

•  Move your head (or imagine moving): does the sound field stay stable?

•  Note: in binaural, the sound field does NOT track head movement (no head-tracking here)

### Step 3 — Encode a mono source  (10')

•  Add a mono file to a new track. Load ICST AmbiEncoder_64. Route to HOA Bus.

•  Set azimuth 0, elevation 0, distance 0 → listen: it should be directly in front.

•  Move azimuth to 0.25 (right), 0.5 (rear), 0.75 (left). Hear it move.

•  Run icst_ambi_encoding_voice.lua: azimuth -45 deg, elevation +20 deg.

### Step 4 — Train flyby automation  (10')

•  Run icst_ambi_zug_langsam.lua (28 s slow train)

•  Then run icst_ambi_zug_schnell.lua (4 s fast train). Compare speed perception.

•  Optional: run icst_ambi_kopter.lua (18 s helicopter — watch elevation change)

### Step 5 — Free encoding and spatialisation  (20')

•  Choose your own audio source (field recording, instrument, synth)

•  Encode it, position it, move it — manually or with a Lua script

•  Add reverb on the HOA Bus. Experiment with distance (Param 13).

•  Partner check: let your neighbour listen — can they identify the position?

### Step 6 — Render B-Format master  (10')

•  File → Render → HOA Render Bus. WAV, 48 kHz, 32-bit float, 16 channels.

•  Open the rendered file in a new session and verify binaurally.

•  Check: 16 channels (B-Format), not 2 channels (decoder output).

# G  Composer Discussion  (45')  —  16:15-17:00

A guided discussion to consolidate the workshop experience and open up compositional and aesthetic questions. This is not a lecture — participants share their observations and develop ideas together.

## Participant Presentations  (15')

•  Each participant (or pair) plays their result from Step 5 for the group

•  1-2 sentences: What did you try? What surprised you? What didn't work as expected?

•  Group reaction: where does it come from? What does the space feel like?

## Discussion Topics (Themen)  (20')

The moderator uses these questions as starting points — follow where the conversation goes:

Space as compositional material:

•  Can the space itself be the instrument? What happens when the room moves and the source stays still?

•  Is 'natural' spatial perception always the goal? When does artificiality become interesting?

The composer's perspective:

•  How does working in Ambisonics change your compositional decisions compared to stereo?

•  What new questions does a full 360-degree sound field raise that stereo never asked?

•  Is Ambisonics a medium, a tool, or an aesthetic? Can it be all three?

Practical experience from today:

•  What worked immediately? What was harder than expected?

•  What would you want to try next — given more time and hardware?

•  Where do you see Ambisonics fitting into your own artistic practice?

## Q&A and Outlook  (10')

•  Open questions from participants

•  Follow-up workshop: MaxMSP + Csound in depth (Blocks D, E, F)

•  Resources: ambisonics.ch — tutorials, scripts, blog articles, B-Format archive

•  ICST website: icst-ambisonics.github.io — plugins, documentation, updates

# Resources & Downloads

### ICST Website

•  icst-ambisonics.github.io  —  Plugins (AmbiEncoder, AmbiDecoder), documentation, MaxMSP externals

•  ambisonics.ch  —  Tutorials, blog articles, Lua scripts, B-Format example recordings

### Downloads (ambisonics.ch)

•  zylia_recording_example.RPP  —  Pre-configured Reaper session for Zylia ZM-1

•  icst_ambi_encoding_voice.lua  —  Encode voice at azimuth -45 deg, elevation +20 deg

•  icst_ambi_zug_langsam.lua  —  Slow train flyby (28 s, physics-based)

•  icst_ambi_zug_schnell.lua  —  Fast train (4 s, Doppler-like)

•  icst_ambi_zug_vergleich.lua  —  Slow + fast in sequence with pauses

•  icst_ambi_kopter.lua  —  3D helicopter flyover (18 s, configurable height)

•  FuMa_to_AmbiX_FOA.jsfx  —  JSFX plugin: FuMa → AmbiX FOA

•  icst_fuma_to_ambix_foa.lua  —  Lua installer for the JSFX plugin

### External References

•  IEM Plug-in Suite  —  iem.kug.ac.at  (free Ambisonics VST3 plugins: encoder, decoder, binaural)

•  Ambix Tools  —  matthiaskronlachner.com/ambiX  (encoder / decoder / rotator)

•  Zylia ZM-1  —  zylia.co  (microphone and Zylia App)

•  REAPER  —  cockos.com/reaper  (DAW, v7+ recommended)

•  Cabbage Audio  —  cabbageaudio.com  (Csound VST wrapper)